- Hummer H2 Door Panel Removal

Procedure to remove interior door panel Hummer H2 model.

The front and rear door panel removal details with diagrams are as follows:
You require a special too to complete this task of door panel dis-assembly.

The special tool referred is a door panel clip tool remover. You can pick up one of these at the local auto parts store. When you use the tool slide it underneath the door panel a give it a quick snap to release the door panel clips. See the door panel removal tool image shown below:

Now the detailed procedure is as follows :

They are the same on front and the rear as far as the procedure goes.

Front Side Door Trim Panel

- Removal Procedure

- Remove the door handle bezel.

- Refer to Front Side Door inside Handle Bezel Replacement diagram shown above.

- Remove the door panel retaining screw located in the handle bezel opening.

- Remove the door window switch.

Use special tool J 38778 to release the door trim panel retaining clips.
Disconnect the electrical wiring harness connectors.
Remove the door trim panel from the vehicle.

- Installation Procedure

Connect the electrical wiring harness connectors.
Install the door trim panel to the vehicle. 
2.1. Align the door trim panel retaining clips with the openings in the door. 
2.2. Push inward on the door trim panel to engage the fasteners. 
Install the door window switch
Install the door panel retaining screw within the handle bezel opening.

Install the door handle bezel. Refer to Front Side Door inside Handle Bezel Replacement

Front Side Door Inside Handle Bezel Replacement

- Removal Procedure

Remove the screw that retains the front inside handle bezel to the door panel.

Remove the bezel from around the door handle,

Remove the bezel from the vehicle

Installation Procedure

Install the bezel to the vehicle.

Install the bezel around the door handle.

Install the screw that retains the handle bezel to the door panel.

Door Lock and Side Window Switch Replacement - Driver Side

- Removal Procedure

Use a suitable flat bladed tool to remove the driver door lock and side window switch from the door trim panel.

Remove the driver door lock and side window switch from the connector.

Installation Procedure:

Install the driver door lock and side window switch to the connector.

Align the driver door lock and side window switch with the opening in the door panel and press to seat the switch.

Rear Side Door Trim Panel Replacement

- Removal Procedure

- Remove the upper door trim panel. Refer to Upper Extension Trim Panel Replacement .

- Remove the rear door handle bezel. Refer to Rear Door Inside Handle Bezel Replacement

- Remove the door panel retaining screw located in the handle bezel opening.

- Use J 38778 to release the retaining clips from the door trim panel.

- Disconnect the electrical wiring harness connectors.

- Remove the door trim panel from the vehicle.

- Inspect for missing or broken retaining fasteners. Replace the fasteners as necessary

- Installation Procedure

- Connect the electrical wiring harness connectors.

- Install the door trim panel to the vehicle.

- 2.1. Align the door trim panel retaining clips with the openings in the door.

- 2.2. Push inward on the door trim panel to engage the fasteners.

- Install the door panel retaining screw.

- Install the rear door handle bezel. Refer to Rear Door Inside Handle Bezel Replacement .

- Install the upper door trim panel. Refer to Upper Extension Trim Panel Replacement .

Upper Extension Trim Panel Replacement

- Removal Procedure

- Firmly grasp and gently pull to disengage the retaining clips.

- Inspect for broke or missing retaining clips and replace as necessary.

Installation Procedure

- Align the retaining clips with the openings in the body.

- Press to fully seat the retaining clips.

Rear Door Inside Handle Bezel Replacement

- Removal Procedure

- Remove the screw that retains the rear inside handle bezel to the door panel.

- Remove the bezel from around the door handle.

- Remove the bezel from the vehicle.

- Installation Procedure

- Install the bezel to the vehicle.

- Install the bezel around the door handle.

- Install the screw that retains the handle bezel to the door panel.